Project Name:

South West Roads Project:

Western Europe-Western China International transit Corridor

CAREC 1b&6b

Kyzylorda Oblast

Lot n.11

SWRP -1917-1948-ICB(W)-001/2009

SITE SPECIFIC ENVIRONMENTAL MANAGEMENT PLAN

(SSEMP)

Table of Contents

0	Executive Summary	3

1	Introduction	8

2	Project Descriptions	10

3	Legaly Policy Fremewok and Clearance Requirements	13

4	Description of Environment	15

5	Anticipated Environmental Impact and Mitigation Measure	18

7	Environmental Monitoring Plan	26

8	Environmental Managmet System	29

9	List of Annexes	39

Table of revision

| rev. n | date | Description | rif. par. | rif. pag. |
| --- | --- | --- | --- | --- |

| 00 01 | 15/04/2010 25/04/2010 | First issued Second issued | All All | All All |
| --- | --- | --- | --- | --- |

# Executive Summary

Background

This is the second revision of the SSEMP.

The SSEMPS that has been submitted by “Todini Costruzioni Generali S.p.A” the first time on March 16, 2010 was not approved by the Supervision Consultants.  The second submission of the SEEMP (rev.00 dated 15/04/2005) was rejected  as well.

This last revision takes in to consideration all the instructions issued till now by the SNC LAVALIN International as PMC of the Project.

Introduction

In this chapter are reported the scope and purpose of this document as well as a brief description of whole Project, the need of site specific EMP, magnitude and location of the Project

Project Description

The lot n. 11 is located in the Kyzylorda Oblast and starts-form  km 1917, passes through the Shieli Region,  and finishes at km 1948 of the road “Border of the Russian Federation (to Samara) – Shymkent via Uralsk, Aktobe,”. The total design length is about 31  km.

The alignment of the new road will match the existing road from Km 1917 to 1922 and form km 1946 to 1947. Instead from Km  1919 to 1942 Km will be constructed a bypass in order to decrease the traffic near the urban area of Shieli

The condition of the existing pavement road is satisfactory for 63% and unsatisfactory for 37%. The main defects in the pavement are bitumen corrugation up to 40%, bumping and heaves are mainly available in the edge part of the pavement, edginess is available almost for 70% out of total length of the road, potholes – from 5% up to 20%, in man sections the road was several times patched, and surface treatment has been done.

Moreover, the life time of the pavement is apparently more than 20-30 years, and during this period only patching and surface treatment in some places has been done.

Legal Policy framework and Clearances requirements

A legal Policy framework of Kazakhstan and applicable permits required for Project construction regarding water quality, soil standard, noise and vibration allowable level, air quality and the Kazakhstan Environmental Code The most important environmental laws and regulations are:

- Environmental Code "Development of Environmental Impact, assessment (EIA)

- Water Code of the Republic of Kazakhstan N° 481 dated July 09, 2003

- SanPiN (sanitary rules and norms) 2.1.4.1 175-02 “Sanitary protection of water sources”.

- “Sanitary and Epidemiological Requirements for the Surface Waters Protection Against the Pollution “

- SanPiN (Sanitary Rules and Norms) 2.1.7.1287-03 Sanitary and Epidemiological Requirements for Quality of Soil and Subsoils

- “Standards of the Maximum Allowable Concentrations of the Hazardous Substances, Harmful Microorganisms and Other Biological Materials Being the Soil Pollutants”

- SNiP (construction norms and rules) 11-12-77 “Noise Protection”.

Description of the Environment

The environmental conditions for the project are described in detail in chapter 4. The project area is characterized by arid climate, sparse vegetation, few year-round surface water courses and large areas with naturally hyper-saline soils. Saksaul forests, which are adapted to dry, saline conditions with extreme temperature differences, play an important role in soil stabilization and erosion control but are not found near the project corridor, nor the bypass alignments. The landscape in the northern project sector is very arid, barren, hardly vegetated and prone to wind erosion, dust generation and moving sand dunes. Surface drainage exists mainly seasonally, when flash floods can occur and draining waters can have a high erosion potential. The landscape has a very soft relief with wide valleys and basins, separated by slightly elevated plateaus. Land use is restricted to low density animal grazing in the natural environment (mainly camels, sheep, goats and some cattle). Permanent settlements are extremely sparse. South and east of Zhosaly the climate is less severe and the settlements are more common, usually clustered around former state farms and railroad facilities. The steppe vegetation, dominated by grassland with small patches of forest near rivers and in valleys, is interrupted by large tracts irrigated with water from the Syr Darya River. . The surface water resources in the project area are dominated by the Syr Daria river, which flows in a NW direction draining into the Aral Sea. Anyway e this river is generally a long distance away from the alignment. The drainage network is sparse in perennial natural streams and rivers, there are relatively few tributaries crossed by the alignments, which flow mostly in SW directions towards Syr Daria. The  artificial irrigation canals form the major features of the surface drainage network.  Groundwater resources along the alignment are usually shallow, near surface aquifers in loose sediments such as sand and gravel. Some aquifers are used for irrigation, human and animal consumption, but many are naturally highly saline or already negatively impacted by anthropogenic activities, such as irrigation and intense agriculture. Thus most near surface aquifers are highly mineralized and high in salinity, as well as being impacted by diffuse pollutants from agriculture.

Anticipated Environment Impact and Mitigation Measure

Chapter 5 of this SSEMP deals with environmental impacts and corresponding mitigation measures. The environment along the alignment of lot n. 11 is not sensitive or particularly valuable in terms of biodiversity and ecological significance. Most of the land is arid steppe to semi-desert landscapes with few river crossings, limited wetlands (partly natural, partly irrigated lands) and no forests, sensitive natural habitats

nor protected areas directly impacted by road construction. An initial review commissioned by the Government and complemented by this study suggests there are no known sites or structures of cultural significance affected by the planned civil works

There are no protected areas directly impacted by the alignment, and only few sensitive natural habitats, rivers, wetlands, forests or protected areas might be indirectly affected. No protected or endangered species will be harmed.

The major part of construction works will remain confined within the existing right-of-way. Thus the Project's investments do not pose unprecedented or significant adverse impacts on the environment that cannot be mitigated.

Environment Monitoring Plan

This plan identified the parameters, indicators, frequency, sampling, time of monitoring activities

Environment Management Plan

This is a specific plan, which might be used as a separate document, according to which all the works will be carried out and contains the instruction in case of occurred emergency. In particular are reported the Todini Environmental Policy and Certification, Responsibilities, site environmental induction, toolbox talks, emergencies and complain

Definitions

| Environment | The term environment refers to an organization’s natural and human surroundings. An organization’s environment extends from the organization itself to the global system, and includes air, water, land, flora, fauna, as well as human beings. |
| --- | --- |
| Environmental aspect | An environmental aspect is a feature or characteristic of an activity, product, or service that affects or can affect the environment. |
| Environmental impact | An environmental impact is a change to the environment. Such change can be positive or negative. Environmental impacts are caused by environmental aspects. Your environmental aspects can have a direct and decisive impact on the environment or contribute only partially or indirectly to a larger environmental change. |
| Environmental management system (EMS) | An organization’s environmental management system (EMS) is one part of a larger management system. The EMS part of this larger management system is used to establish an environmental policy and to manage the environmental aspects of an organization’s activities, products, and services. A management system is a network of interrelated elements. Elements include responsibilities, authorities, relationships, functions, processes, procedures, practices, and resources. A management system uses these elements to establish policies and objectives and to develop ways of applying these policies and achieving these objectives. |
| Environmental objective | An environmental objective is a specific environmental goal. Your organization’s environmental objectives must be consistent with its environmental policy. |
| Prevention of pollution | To prevent pollution means to avoid, reduce, or control the creation, emission, or discharge of contaminants or waste materials. Pollution must be prevented in order to reduce adverse environmental impacts. |

Referring  to other definitions is to be made to the terms and definitions in the ISO 14001 regulation.

Abbreviations

| EMS | Environmental Management System |  |
| --- | --- | --- |
| PEA | Preliminary Environmental Analysis |  |
| ESIA | Environmental and Social Impact Assessment |  |
| EE | Ecological Expertise |  |
| EARF | Environmental Assessment Review Framework |  |
| EMP | Environmental Management Plan |  |
| NC | Non Conformity |  |
| PA | Preventive Action |  |
| EP | Environmental Procedure |  |
| TEG | Technical Environmental Guidelines |  |
| MSDS | Material Safety Data Sheet |  |
| AM | Area Manager |  |
| PM | Project Manager |  |
| PMC | Project Management Consultant |  |
| CEM | Contractor Environmental Manager |  |
| PLM | Plant Manager |  |
| GSM | General Service Manager |  |
| PRM | Purchase Manager |  |
| CM | Construction Manager |  |
| SM | Safety Manager |  |
| EI | Environmental Inspector |  |
| DOE | Department of Environment (oblast level) |  |
| EBRD | European Bank of Reconstruction and Development |  |
| MOEP | Ministry of Environmental Protection |  |
| MOTC | Ministry of Transport and Communication |  |
| RC | MOTC’s Roads Committee |  |

# Introduction

  - Brief Description , Magnitude and Location of the whole Project

The  whole Project covers the area between the cities of Shymkent and the border between Kyzylorda Oblast and Aktobe Oblast. The section of the Project of whose Todini is the Main Contractor is constituted by six lots located in the Kyzylorda oblast (approximately 220 km) starting form km 1837, just outsided the urban area of  Kyzylorda, till the border with Shymkent oblast:

| Lot Number | From Km | To Km | Extension Km |
| --- | --- | --- | --- |
| 9 | 1837 | 1877 | 40 |
| 10 | 1877 | 1917 | 40 |
| 11 | 1917 | 1948 | 31 |
| 12 | 1948 | 1980 | 32 |
| 13 | 1980 | 2012 | 32 |
| 14 | 2012 | 2057 | 45 |

In particular this SSEMP has as subject the lot n 11.

For this section most of the alignment entails widening of the road from a two lane cross section to four lanes, and the construction of several large bypasses and structures such as flyovers and bridges and has been classified as Category 'A'

The proposed alignments for the road sections, including the bypasses, were examined in detail by the Project team during a series of field missions that  has includ environmental and social specialists. The majority of the construction works,will remain confined within the existing right-of-way thus the Project is not expected to have unprecedented or significant adverse impacts on the environment that cannot be mitigated.

The environment along the alignment is neither sensitive nor particularly valuable in terms of biodiversity and ecological significance. Most of the land is arid steppe to semi-desert landscapes with few river crossings, some wetlands (partly natural, partly irrigated lands) and no forests, sensitive natural habitats or protected areas directly impacted by road construction.

According the environmental Screening exercise conducted by the World Bank, the lots, from km 1917 to 2057,  present a medium environmental risk.

  - Need and Object of SSEMP

This SSEMP was elaborated in order to observe the Clause N. 4.18 of part B – Specific Provision “Environment Protection”. This plan is one of the six plans prepared for being used by all Todini Costruzioni Generali and Subcontractors staff involved in the section where Todini is the Main Contractor .

This plant will refer and conform with the following documents and instructions :

- Environmental Procedure PA 08 of Todini’s Environmental Management System.

- Tender Specification, General and Specific Provisions of the lots contracts nr. 11, 12, 13 and 14 of “South West Road Project Western Europe-Western China International transit Corridor”.

- Environmental & Social Impact Assessment of February 2009.

- Manual and Requirements for SSEMP structure and details, attached to the letter of SNC Lavalin International, dated on 15 May, reference SWRP/KZ/TL/SC/5-10/117.

- Comments of the PMC Environmental Supervisor of Lavalin SNC International.

- World Bank Policy and Procedures.

The content of the SSEMP is applicable to all the activities, occurred during the construction period, by wich Todini can be considered responsible and which regarding the contract nr 11 SWRP -1917-1948-ICB(W)-001/2009

The general scope of the SSEMP is:

- to establish Todini Site Organizations’ and Subcontractors’ duties and responsibilities regarding the generation of environmental impacts, the application of the mitigation measures;

- to arrange a  procedure to follow in case of  occurred emergencies or complain;

- to establish the way for conducing the monitoring and the site environmental inspections;

- to give a legal framework in order to avoid any breach of environmental regulatory of Kazakistan.

Where the content of this Plan is perceived to be inadequate or the construction activities change from the original contract scope, a revision of the SSEMP will be required to address the activity changes.

The Contractor’s Environmental Manager is responsible for distributing revised versions of the SSEMP to all copy holders.  The Project Manager is responsible for ensuring that the site staff and subcontractors will aware of any changes to the SSEMP.

This EMP does not address any health and safety matters.  For advice on health and safety matters, refer to a suitably qualified and experienced safety professional.

# Project Description

  - Lot number 11: existing road and proposed project

The lot n. 11 is located in the Kyzylorda Oblast and starts-form  km 1917, passes through the Shieli Region,  and finishes at km 1948 of the road “Border of the Russian Federation (to Samara) – Shymkent via Uralsk, Aktobe,”. The total design length is about 31 km.

The alignment of the new road will match the existing road from Km 1917 to 1922 and from km 1946 to 1947. Instead from Km  1919 to 1942 Km will be construct a bypass in order to decrease the traffic near the urban area of Shieli Section of the existing roadkm passes in south-eastern direction on the right side coast of the river Syrdarya, where the surface formed by sand was exposed to wind processing. Thus, ridges, dunes, salty depressions and hollows were formed. When crossing these sections it can be  seen  that the longitudinal profile is slightly wavy.

In principal the road has a new direction: 23km out of total 29 km. The thickness of the existing asphalt concrete pavement road being used is 10-25cm. Condition of the pavement is satisfactory for 63% and unsatisfactory for 37%. The main defects in the pavement are bitumen corrugation up to 40%, bumping and heaves are mainly available in the edge part of the pavement, edginess is available almost for 70% out of total length of the road, potholes – from 5% up to 20%, in man sections the road was several times patched, and surface treatment has been done.

The main reason for defects at many sections of the pavement is construction of pavement base course from the soils with low bearing capacity as well as from pebble and gravel soils of small capacity (0 – 25 look the scheme of thickness measurement). Moreover, the life time of the pavement is apparently more than 20-30 years, and during this period only patching and surface treatment in some places has been done. Compaction factors are 0,96-0,98. Density of pebble soil of the pavement base course is 2,16-2,32 t/cm3, maximum dry density is 2,2-2,40 t/m3. Thickness of pebble soil is within 0 cm to 25cm. Embankment of the existing road is done mainly from fine sand and light loam, heavy dusty loam and relatively under compacted.

Embankment of the existing road is mainly done from the soils from the borrow pits along the road.

Foundation of embankment and road side is made of sands and loams, in the lower sections one can meet clay.

Scope of the Contract stipulates reconstruction of 29km of the motor road. Scope of the Contract provides reconstruction of the motor road, providing for reconstruction of the motor road of the II technical classification converting it into Ib technical classification, construction of new embankment at the section of alignment, milling of the existing pavement for the whole thickness and width (then the material to be used at the site), construction of new pavement: subbase layer from sand-gravel mixture; bottom layer of the base course made of crushed stone-sandy mixture С4; upper layer of the base course made of high-porous asphalt concrete layer and asphalt concrete binder course and Stone Mastic Asphalt as wearing course; protection of shoulders; construction of bridges and overpasses; construction of new culverts; road furniture and road marking; traffic safety measurements and measurements on reduction of environmental impact.

  - Facilities

An Asphalt Plan is planned in the lot n. 11 at Km 1937 on the right side of the road (about 200 m).

Meanwhile, along the way, will be some borrow pit in order to extract the suitable material for the embankment and granular pavement.

A list of the above mentioned Borrow Pit is included in the Annex n.1.

  - List of material Flow

| Description | Unit | Quantity |
| --- | --- | --- |
| Concrete drainage channels in shoulder | пм | 2.489 |
| Concrete drainage channels down embankment side slope | пм | 456 |
| Subbase material | m3/м³ | 152.588 |
| Shoulder material | m3/м³ | 109.004 |
| Crushed stone base course | m3/м³ | 127.231 |
| Tack coat | m2/м² | 1.051.261 |
| Top layer of crushed sone base course (highly porous asphalt) | m3/м3 | 66.886 |
| Asphaltic base course | m3/м3 | 52.563 |
| Asphaltic base course for bridges | m3/м3 | 95 |
| Wearing course | m3/м3 | 4.329 |
| Wearing course for bridges | m3/м3 | 442 |
| Stone-mastic asphalt | m3/м3 | 26.282 |
| Bituminous mastic | l.m./пм | 48.157 |
| Precast reinforced concrete pile driving, B25 F200 | m3/м³ | 960,8 |

# Legal Policy Framework and Clearance Required

  - Legal Framework

Esia and Licenses

- According to Article 36 of the Environmental Code "Development of Environmental Impact, assessment (EIA) is obligatory for all types of activities that can have a direct or indirect impact on the environment or health of the people

- Another process regulated by the Environmental Code is the  Environmental Audit. The audit is mandatory when an enterprise legal person significantly damages the environment;

- A license is compulsory before starting operations for that activities reported in the list

Air quality standard

- The emission of the hazardous substances (pollutants) in the atmospheric air by the stationary source is allowed only on the basis of the permit issued by the authorized state body in the field of atmospheric air protection or its territorial subdivisions in the manner established by the Government of the Republic of Kazakhstan.

Water quality standard

- The main legislative act in the area of water resources protection and use is the Water Code of the Republic of Kazakhstan N° 481 dated July 09, 2003

- Article 116 of the Law regulates issues related to the water protection zones: to maintain water bodies and water facilities in the condition required by the hygiene and sanitary and ecological norms; to prevent contamination, blockage and depletion of the surface water; to preserve flora and fauna water protection zones and belts are required.

- The requirements for quality of water of the non-central water supply are defined by the SanPiN (sanitary rules and norms) 2.1.4.1 175-02 “Sanitary protection of water sources”. The smell, flavor, color of water, turbidity and coli index are normalized at that and it is specified that the content of the chemical substances shall not exceed the values of the relevant standards.

- The basic document regulating the condition of the surface waters and content of the hazardous substances in them is the sanitary and epidemiological norms and regulations “Sanitary and Epidemiological Requirements for the Surface Waters Protection Against the Pollution “ NO 3.dd 02.03.04 approved by the Order of the Ministry of Health of the RoK NO 506 dd 28.06.2004.

- The effluent of the hazardous substance (pollutants) in the surface water resources is allowed only on the basis of the permit issued by the authored state agency in the field of water bodies' protection or its territorial subvision in the procedure determines by the Government of Republic of Kazakistan.

Soil Standards

- The SanPiN (Sanitary Rules and Norms) 2.1.7.1287-03 Sanitary and Epidemiological Requirements for Quality of Soil and Subsoils establish the specifications for soils quality in the inhabited localities and agricultural lands and control the observance of the sanitary-hygienic standards during location, engineering, construction, renewal (technical upgrading) and operation of the facilities of different purposes, including those which may cause the adverse effect on the soils

- The basic regulatory document for control of the soil pollution contentis “Standards of the Maximum Allowable Concentrations of the Hazardous Substances, Harmful Microorganisms and Other Biological Materials Being the Soil Pollutants”

Noise standards

- The level of the road traffic noise is determined according to the norms of the SNiP (construction norms and rules) 11-12-77 “Noise Protection”.

  - Clearances required

Currently all the licenses for the borrow pits are in progress as well as the license for Asphaltic Plan.

# Environmental Description

  - Location

Kyzylorda oblast is located in the Southern part of the republic, along the lower flow of the Syrdarya River, on the coasts of the Aral Sea. The oblast borders on Uzbekistan in the Southwest. The territory of the oblast occupies 226 thousand sq. km. There are only 628.6 thousand people living on the enormous territory of Kyzylorda oblast, i.e. 2.8 people per 1 sq. km., which is 2 times below the average figure in the country. The oblast center is Kyzylorda city, where lives 32.4% of the oblast population. From 1925 to 1929 the city of Kyzylorda was the capital of Kazakhstan. There are 7 rayons, 3 cities, and 287 settlements and villages in the oblast.

  - Relief

The region is located in the belt of Asian deserts, along the lower flow of the Syrdariya River. It is located in the extensive Turanian lowland with the plain relief, the largest part which is the ancient-delta plain of the Syrdariya, Sarysu, and Chu rivers. Absolute marks of the plains vary from 200m in the Southeast to 53m in the West, on the shore of the Aral Sea. Southwestern slopes of the western part of the Karatau mountains - which are the part of the Tien Shan mountains – are in the Eastern part of the region. Absolute marks of the range of mountains vary within 400-600m, the maximum height is 1418m.

There are extensive (individual sections in the North) massifs of sands with cellular and uneven structure in the South and the West of the oblast.

  - Climate

The climate of the region is sharply continental: hot dry summer and cold winter with unstable snow cover. The average annual temperature in the region composes +7-11°. The annual amplitude of the temperature (the difference of the average temperature of the warmest and coldest months) varies from +34 to - 41°.

The absolute maximum temperature composes 42-46°, minimum - 34- 39°. The region is very arid. Precipitation level reaches 95-159mm per year, in separate dry years can fall out only 30-60mm, and during the most moisturized period - 200-2 13mm.

Average annual wind speed does not exceed 3-4 m/s. Wind speed reaches its maximum during spring and winter months. North-eastern and North-western winds predominate in the South-east. Dusty storms are observed during warm seasons.

  - Natural zones

Zone of hot dry desert steppes and deserts: divided into three subzones: a) Northeastern, b) Central, c) Southeastern.

The first and second subzones are characterized by the watered agriculture (rice) and cattle breeding. The third zone is good for cattle breeding.

  - Soil cover

Soils of the region vary significantly but are distinctly subdivided into two large regions: the region of the moistened (hydro-morphic) soils and dried up (sub-aerial) soils with signs of ancient irrigation of the soils in desert zones.

The highway route displays the following soils: brown with desert alkali soils, sands in combination with brown soils, grayish-brown soils; dry-type playa soils in combination with sands, and pure sands.

As mentioned above, the soils of region are extremely diverse with the exception of sandy soils and soils which are formed by tertiary and chalky deposits and grayish-brown soils. The soils possess the following general features: high carbonate content; layered structure of soils; absence of the microstructure and waterproof microstructure; saltiness (salted chloride-sulfate with dominating sulfates of sodium and calcium); strong biogenic processes which lead to rapid decomposition of humus and loss of the fertility of soils.

The content of humus varies from 0.6 to 3%. As soil-forming species serve sandy loam, loams, clays, and sand. Fertile soil cover is subject to removal for 15cm on grayish-brown soils.

  - Surface and ground waters

According to hydrological division Kyzylorda oblast refers to the ponds of the internal drain of Central Asia and Kazakhstan. The drainage network of the region is represented by the Syrdaraya River which flows into the Aral Sea and by temporary water flows (Besaryk, Tules, Zhideli, Akuyuk, Shulak, and others) which exist during the flood period. The Syrdariya River forms many lakes (Zhaksykylysh, Kamystybas, Arys, and Ashikol) on its way along the plain. These lakes are called oxbows.

Ground waters lie at the depth of 2-7m. Ground water of dry-type playa plains lie at the depth of 10-30m and strongly mineralized. Besides ground water there are many artesian sources. Wells of 5-50 l/s predominate.

  - Vegetation

Vegetation of the region is diverse: erekovo-wormwood, black saxsaul, kuyreuk, zhantak, byurgun, licorice, couch-grass, adzherek; from wood species - oleaster, willow, poplar, tamarisk, ephemer.

  - Settlement

There are several little settlements  which are located along the road. The main ones are listened below:

| PK | Name of Settlement | Location (designed road) |
| --- | --- | --- |
| 1936+900 - 1940+700 | Shiyeli s. | On the right – 1.7 Km |
| 1935 | Aktorgan | On the left along the way |
| 1942-1943 | Tonkeris | On the right along the way |
| 1946 | No name | On the rightt 900 m |

All the settlement are marked in the General Plan included in the Annex 1.

# Anticipated Environmental Impact and Mitigation Measures

  - The Significant Environmental Impacts

The Significant Environmental Activities has been figured out from the analyses of all the environmental aspects, regarding the Site activities and works, are included in two different document: the EIAS and the PEA. The first document was prepared by Ministry of Transport and Communication according to the Kazakhstan Environmental Code. Instead the second one was prepared by the Todini HSE Main Office located in Roma according the PA 01 of  Todinis’ EMS.

The following table reports the Significant Environmental Aspects coming from both the above mentioned studies.

Table of Direct Significant Environment Aspect

| Significant Environmental Aspects | Significant Environmental Impacts | Type of aspect | Location /Source |
| --- | --- | --- | --- |
| Water Consumption (WC) | Sustainable Development | Domestic use/Potable | Allocations |
|  |  | For Work | Concrete Casting- Ground mixing water- Washing Machines and equipment- Concrete Curing |
|  |  | Mitigation Measure | Dust Abatement |
| Energy Consumption (EC) | Sustainable Development | Diesel/Gasoline | Equipment (work vehicles and asphalt plants) |
|  |  | Electricity | All Facilities |
| Inert Consumption (WMC) | Sustainable Development | Aggregates and Sand- | Asphalt Plan |
|  |  | Aggregates and Sand | Concrete Plan |
|  |  | Inert | Base and Subbase |
| Chemical Substance Consumption (WMC) | Sustainable Development | Bitumen-Addictive | Asphalt Plan |
|  |  | Addictive and Cement | Concrete Plan |
| Production of Waste Material (WP) | Likely pollution of soil, air and water | Waste Oil, Air and Oil filters, Drums, Batteries, Tires, adsorbent material | Workshop |
|  |  | General Waste | All facilities |
|  |  | Construction Waste (Concrete, wood, asphalt and steel) | Site Clearance-Demolition and Milling- Precasting center |
|  |  | Containers | Use of chemical substances |
|  |  | Packaging and expired materials | Store |
|  |  | Sewerage Water | Septic tank |
|  |  | Unsuitable Soil | Excavations |
| Discharge on the ground (DGW) | Soil and water pollution | Fuel | Refueling |
|  |  | Chemical Substance | Use and storage of liquid chemical substance-Dripping or linking from vehicle |
|  |  | Contemned water | Washing machines and Equipments |
|  |  | Hazardous materials stocked on the ground | Waste material, steel etc. |
|  |  | Cemented Water | Concrete Curing |
| Discharge in the air (DA) | Air Pollution | Dust | Aggregates and soil deposit, Transport of soil and sand, Concrete and asphalt plan, Crushing plan, Traffic. |
|  |  | Exhausted fumes | Vehicles and Equipments |
|  |  | Organic Substance | Manufacturing, tasport and use of asphalt |
| Noise and Vibration | Damages of Building. Nuisances for human activities and local fauna | Knocking | Heavy Vehicles and Equipments, Heavy Traffic, load and unload of loose material, |
|  |  | Blasting | Barrow Pit |

Further the direct impacts, we have to consider the indirect impact. The activities, not directly managed by Todini Company, which can generate significant environmental impacts are:

- Transport of material from other Company to Site. (TOM)

- Production of construction products (pipes, chemical products, precasting, etc)  in other companies. (PCP)

- Transport and dispose of waste material produced by Site activities.(DWM)

  - Mitigation Measures

All Todini Staff and the Subcontractors, in order to minimize the environmental impact generated by the Site activities, will adopt the following measures:

| CODE | ENVIR. ASPECT | MITIGATIVE MEASURES | LOCATIONS | RESPONSABILITY |
| --- | --- | --- | --- | --- |
| WP 1 | Waste Production | Materials will be purchased in the exact quantity required for the contract, to minimize unused left-over; | Store and Site | PRM |
| WP 2 | Waste Production | Materials that with minimal packaging will be purchased, to minimize packaging thrown out | Store and Site | PRM |
| WP 3 | Waste Production | Materials will be reused on site where possible | Site | CM |
| WP 4 | Waste Production | The following waste will be recycled where it is possible: cardboard, paper, metal re-bar, scrap metals such as soft-drink cans, used batteries from vehicles/ plant, waste oil, drained oil filters; | Workshop Office Site | PLM GS CM |
| WP 5 | Waste Production | Waste will be stored in containers or skip bins, and not stockpiled directly on unsealed ground | All Facilities | CM PLM Office Chief |
| WP 6 | Waste Production | Waste removal from the site will be scheduled so that there is always a waste skip available for use on site, and that waste skips/ container do not overfill | All facilities | GSM |
| WP 7 | Waste Production | Any temporary waste storage areas (not contained in bins or containers) will be covered and/or surround by a screen mesh fence to prevent it being wind-blown across the site and into waterways and adjacent property | Site | CM |
| WP 8 | Waste Production | Liquid wastes (excluding sewerage waters) including waste oil and liquid chemicals will be stored in sealed drums/ containers, on a concrete surface. | Workshope and Site | CM PLM |
| WP 9 | Waste Production | All waste removal records will be maintained, and reported. Records that will be kept include: receipts/ invoices/ dockets from the waste transport contractor and the waste receiving facility; and these records will be kept in a Waste Register, which records collection dates, type of waste, quantities, waste transport company, destination, and signature of an authorized person. | All Facilities | CM GSM PLM |
| DA 1 | Discharge in the Air | All equipment, generators, machinery and vehicles used on site will be maintained in good working condition at all times to ensure minimal fuel consumption and smoky exhausts. ; | All equipments | PLM |
| DA 2 | Discharge in the Air | Unsealed roads and work areas will be sealed as required, to control dust | Site | CM |
| DA 3 | Discharge in the Air | An adequate water supply will be provided for dust suppression during earthworks and Stockpiles of material or debris shall be dampened prior to their movement, except where this is contrary to the Specification. | Site | CM |
| DA 4 | Discharge in the Air | Truck hauling fill or other dusty materials will be covered, and loads will be kept 0.3 meters below the top of the truck walls, with a sheet or tarpaulin to control dust; | Site | CM |
| DA 5 | Discharge in the Air | Vehicle speeds will be minimize to control dust generation | Site | CM |
| DA 6 | Discharge in the Air | Stockpiles of materials shall be sited in sheltered areas or within hoarding , away from sensitive area Stockpiles of friable materials shall be covered with clean tarpaulin, with application of sprayed water during dry and windy weather. | Site Plants | CM PLM |
| DA 7 | Discharge in the Air | Painting, abrasive blasting, metal cutting, grinding or welding will be done within an enclosed and ventilated area wherever possible, to limit overspray or the discharge of volatile substances, fumes and dust/grit suspension into the atmosphere | Site | CM |
| DA 8 | Discharge in the Air | Diesel, oil, paint, thinners and other chemicals used on the site will be kept in minimum quantities and stored in sealed containers and in a manner that limits vapors, and in accordance with any safety requirements | Site Workshop Store | PLM CM STM |
| DA 9 | Discharge in the Air | The Discharge directly in air have to respect the allowable limits as reported in the national Law | Plants | PLM |
| DA 10 | Discharge in the Air | Dry cement powder will be kept in sealed bags or containers to prevent dust generation | Site/ Concrete Plan/ | CM PLM |
| DA 11 | Discharge in the Air | All the devices for the abatement of hazardous substance in the air have to be regularly maintained and always working | Plants | PLM |
| DA 12 | Discharge in the Air | The feature of the used fuel has to comply with specification of engines of vehicle and road equipment | Store | PLM STM |
| DA 13 | Discharge in the Air | The dirt road, access of open pits will be periodic moistening by water | Site | CM |
| DA 14 | Discharge in the Air | Open Burning is Prohibited | Site Plants | CM PLM |
| DA 15 | Discharge in the Air | Stockpiles of materials shall be sited in sheltered areas or within hoarding , away from sensitive area | Site Plants | CM PLM |
| DA 16 | Discharge in the Air | Stockpiles of friable materials shall be covered with clean tarpaulin, with application of sprayed water during dry and windy weather. | Site Plants | CM PLM |
| DSW 1 | Discharge on ground or into water | Concrete residue will only be washed out into a properly designed and managed wash-out bay | Site | CM |
| DSW 2 | Discharge on ground or into water | Diesel, oil and other fuels and liquid chemicals will be kept in sealed containers, drums or tanks. The storage area will be located away from or wetlands any watercourse | Site Workshop Plants Store | CM PLM SM |
| DSW 3 | Discharge on ground or into water | Drums and containers used for fuel or liquid chemical storage (including waste oil, paints, thinners) will be in good condition and free from rust or damage | Site Workshop Plans Store | CM PLM SM |
| DSW 4 | Discharge on ground or into water | An MSDS will be available in the site office for all chemicals used on site | Site Workshop Plants Store | CM PLM SM |
| DSW 5 | Discharge on ground or into water | Diesel, oil and other fuels and liquid chemicals stored in bulk quantities (500 Litres or more) will be in accordance with Todini EMS. This includes storing drums/ tanks within a bunded area, capable of storing 110% of the largest container in event of a spill, and ensuring bunds will be structurally sound and sealed to prevent seepage | Site Workshop Plants Store | CM PLM SM |
| DSW 6 | Discharge on ground or into water | All fuel and chemical storage will be sited on an impervious base within a bund and security fencing. | Site Workshop Plants Store | CM PLM SM |
| DSW 7 | Discharge on ground or into water | Re-fuelling of cars, trucks and easily mobile vehicles will be done at a designated area, over an impervious concrete pad of sufficient size, so that spills and overflow does not fall onto the ground | Re fuelling Station/ Store | PLM |
| DSW 8 | Discharge on ground or into water | Mobile re-fuelling, i.e. delivery to field equipment/ plant by tanker, will only be done where equipment/ plant cannot be easily returned to a designated re-fuelling area. Prior to starting re-fuelling, a drip tray and ground protective sheet will be placed under the refuelling point | Re fuelling Station/ Store | PLM STM |
| DSW 9 | Discharge on ground or into water | Tankers delivering fuel to field based equipment/ plant must have the following equipment available within the vehicle: drip trays, ground protective sheets; a labeled and sealed container for storing spills | Re fuelling Station/ Store | PLM STM |
| DSW 10 | Discharge on ground or into water | Spill clean-up kits will be kept near areas used for fuel or liquid chemical storage. Staff will receive training in the use of spill clean-up kits | Site Workshop Plans Store | CM PLM SM |
| DSW 11 | Discharge on ground or into water | Hoses used for re-fuelling from bulk diesel storage tanks will be kept within the bunded area or over concrete when not in use | Re fuelling Station/ Store | PLM STM |
| DSW 12 | Discharge on ground or into water | Routine maintenance and repair of mobile equipment/ vehicles must be done only in a workshop | Workshop | PLM |
| DSW 13 | Discharge on ground or into water | The soil reused in the Site coming from excavations or site clearance have to be not contemned and a Register of that soil have to be maintained recording the type of soil, the destination, the quantity, and the origin | Site | CM |
| DSW 14 | Discharge on ground or into water | Sewage holding tanks will be pumped-out as frequently as-required, to prevent over flow. A schedule for regular sewage tanker pump-out of sewage holding tanks will be established. | Labour Camp and Rest Areas | GSM |
| DSW 15 | Discharge on ground or into water | The Discharge directly in the water have not exceed the allowable limits | All Facilities | CEM |
| DSW 16 | Discharge on ground or into water | All valves and trigger guns shall be resistant to unauthorized interference and vandalism and be turned off and securely locked when not in use | Refueling Workshop Plans | PLM STM |
| DSW 17 | Discharge on ground or into water | The contents of any tank or drum shall be marked | Refueling Workshop Plans Site | PLM STM CM |
| DSW 18 | Discharge on ground or into water | The Contractor shall at all times ensure that all exiting stream courses and drains within and adjacent to the Site are Kept safe and free from any debris and any materials arising from the Work | Site | CM |
| NV 1 | Noise and Vibrations | Vehicles and equipment including generators and pumps will be well-maintained, and fitted with mufflers to reduce noise | Workshop | PLM |
| NV 2 | Noise and Vibrations | Noise levels presented will not be exceeded the allowable limits | All facilities | CEM |
| NV 3 | Noise and Vibrations | All noise or vibration complaints will be investigated and addressed appropriately. | All Facilities | CEM |
| NV 4 | Noise and Vibrations | Generators and pumps will be positioned away from sleeping areas in labor accommodation, or other residential areas | Site | CM |
| NV 5 | Noise and Vibrations | The velocity of the traffic flow will be limited to 60 Km/h | Site | CM |
| WC 1 | Water Cons. | If not provided from the public water network, the water will be stored in sealed drums. No sign of dripping or tap broken | All facilities | All |
| WC 2 | Water Cons | The water for construction work will be carried by mobile tank without dripping, leaking and will be regularly maintained | Site | CM PM |
| WC 3 | Water Consumptio | Some activities requiring the water use, as machine washing or abatement of dust , will be carried out only if really necessary. | All facilities | All |
| WC 4 | Water Consumptio | Where possible the water will be recycled. Areas where water is regularly or repetitively used for dust suppression. | Plants Site | Plants Site |
| EC 1 | Energy Consumpti | All electrical device not utilized will be switch off . | All facilities | All |
| EC 2 | Energy Consumpti | The use of alternative and sustainable energy sources will be taken in considerations where economically possible. | All Facilities | PLM |
| EC 3 | Energy Consumpti | All new electrical device purchased will have low energy consumptions | All facilities | PRM |
| WMC 1 | Work Material Consump. | The quantity of material used at the Site for manufacturing construction product like concrete, asphalt concrete, etc., will respect the proportion reported in the project without useless wastes. | Plants Site | PLM |
| WMC 2 | Work Material Consump. | All the product and material will be conserved as prescribed by the Supplier or the Manufacturing Company | Store and Site | STM CM |
| TCM 1 | Transport Const. Material | The transport of construction material to Store or Site will be planned (time and way) and will carry out only during regularly work time in order to avoid nuisance or traffic congestion | Store and Site | CM |
| DWM 1 | Dispose of Waste Material | The company in charge to dispose of any waste material generated at the site, will bring it to a Authorized center and will provide any kind of document to demonstrate the delivery as scheduled | All Facilities | PRM |
| DWM 2 | Dispose of Waste Material | Where possible the Company chosen for the transport and dispose of waste material will be certified ISO 14001 | All Facilities | PRM |
| PCM 1 | Production Of Constr. Material | Where possible will be chosen Company certified ISO 14001. In other case the PM will charge the CEM to carry out a Environmental Inspection | All Facilities | PRM-PM |

# Monitoring Environmental Plan

Todini Site will provide samplings, analyses an tests reported in the further paragraphs and they will be undertaken by third-party organization found acceptable to the Engineer and Environmental Protection Services of the Republic of Kazakhstan. It is responsibility of CEM to files the analyses reports and advise the Engineer in case of non conformity of the parameters trend.

The information and license of the Third Party monitoring firm is are included in the Annex 4.

Additional quality monitoring shall be carried out by the Contractor at locations to be determined by the Engineer if warranted by events.

- Air Quality

Location and setting:

| Code | Lot n. | PK (Km)(*) | Side (**) | Distance from design road (**) m | Point od Sampling (Coordinates) | Name of settlement |
| --- | --- | --- | --- | --- | --- | --- |
| AIR 11-1 | 11 | 1927+400 | LEFT | 210 | 44 °14' 44,94'' N 66 °39' 16,08'' E | AKTORGAN |
| AIR 11-2 | 11 | 1939+200 | LEFT | 400 | 44 °11' 41,28'' N 66 °47' 18,01'' E | TONKERIS |
| (*) | The Kilometers reffer to the design Road |  |  |  |  |  |
| (**) | The Side (right or left) refferS to the direction Kyzylorda-Shymkent |  |  |  |  |  |

Parameters:

| Symbol | Parameter Description | UM |
| --- | --- | --- |
| PM10 | PARTICULAR MATTER | mg/l |
| T | TOTAL SUSPENDED PARTICULATE | mg/l |
| SO2 | SULFUR DIOXIDE | mg/l |
| NO | NITROGEN MONOXIDE | mg/l |
| CO | CARBON MONOXIDE | mg/l |
| CH2 | HYDROCARBUR | mg/l |

Frequency: Once a month x 3 years

Time and Duration of the measure : 6 hour

- Water Quality

Location and setting:

| Code | Lot n. | PK (Km)(*) | Point od Sampling Coordinates | Name of settlement |
| --- | --- | --- | --- | --- |
| WAT 11-1 | 11 | 1929+958 | 44 °14' 01,94'' N 66 °41' 09,37'' E | CANAL PK 161+27,30 |
| WAT 11-2 | 11 | 1940+125 | 44 °11' 49,57'' N 66 °47' 34,54'' E | CANAL PK 161+27,30 |
| (*) | The Kilometers reffer to the design Road |  |  |  |

Parameters:

| Symbol | Parameter Description | UM |
| --- | --- | --- |
| PH@20 C | PH | UNIT |
| T | TEMPERATURES | C |
| / | TUBIDITY | UNIT |
| / | COLOR | UNIT |
| as CaCO3 | ALKALINITY | mg/l |
| TSD | SOLID DISSOLVED | mg/l |
| / | OIL AND GREASE | mg/l |
| BOD | TOTAL ORGANIC MATTER | mg/l |
| OD | OXIGEN DISSOLVED | mg/l |
| as CaCO3 | TOTAL HARDNESS | mg/l |
| as CaCO3 | HARDNESS CALCIUM | mg/l |
| as CaCO3 | HARDNESS MAGNESUM | mg/l |
| Na | SODIUM | mg/l |
| K | POTASSIUM | mg/l |
| HM | HEAVY METAL (Fe, Cu, Zn,Pb, Cd, Ge, Hg) | mg/l |
| / | COLIFORM COLONIES | colony/ 100 ml |
| / | FECAL COLIFORM | colony/ 100 ml |
| / | PHENOLS | colony/ 100 ml |
| NOx | NITROGEN OXIDES | mg/l |
| NO | TOTAL NITROGEN | mg/l |
| / | POLIFOSFORUS | mg/l |

Number of sample to analyze :One per setting

Frequency: Once a month x 3 years

- Noise and Vibration

Location and setting:

| Code | Lot n. | PK (Km)(*) | Side (**) | Distance from design road (**) m | Point od Sampling (Coordinates) | Name of settlement |
| --- | --- | --- | --- | --- | --- | --- |
| NVB 11-1 | 11 | 1927+400 | LEFT | 210 | 44 °14' 44,94'' N 66 °39' 16,08'' E | AKTORGAN |
| NVB 11-2 | 11 | 1939+200 | LEFT | 400 | 44 °11' 41,28'' N 66 °47' 18,01'' E | TONKERIS |
| (*) | The Kilometers reffer to the design Road |  |  |  |  |  |
| (**) | The Side (right or left) refferS to the direction Kyzylorda-Shymkent |  |  |  |  |  |

Parameters:

| Symbol | Parameter Description | UM |
| --- | --- | --- |
| Leq | NOISE LEVEL | db |
| SEL | SOUND EXPOSOURE | db |
| / | FREQUENCY SPECTRUM X,Y,Z | N |

Duration of the measure :  6 hours

Frequency: Once per month x 3 years

# Environmental Management Plan

  - Todini Environmental Policy and Certification

Todini Costruzioni Generali S.P.A is fully aware that a responsible economic strategy targeting the environmental problems derived from its activities is essential to its own and its Customers’ success.

The Company also recognizes that continual improvement of its own environmental performance brings significant commercial and economic benefits, while at the same time satisfying the environmental improvement expectations in the territorial setting where it operates. Therefore, the Company undertakes to pursue a policy of continual improvement of its own environmental performance, by minimizing, where possible or economically sustainable, its activities’ negative impact on the environment.

Todini S.p.A. aims to achieve the goals indicated above through the following actions:

- ensuring that its activities are performed in compliance with the regulations of law in force and with any practise codes that have been signed;

- putting into effect and maintaining an effective Environmental Management System in accordance with the UNI EN ISO 14001 requirements;

- implementing all organizational, operational, and technological efforts to prevent pollution of the air, water, and soil;

- minimizing energy and water consumption and the production of waste, while favouring their recovery wherever possible;

- ensuring that the environmental policy discussed here is documented, implemented, kept active, and communicated to all persons working for or on behalf of the organization;

- ensuring that this document is available to the public.

  - Personnel and Responsibilities

In the following table are reported the general responsibilities of the Todini Site Staff.

| Title | Responsibility |
| --- | --- |
| All Staff (included subcontractors) | - Follow the requirements of this EMP - Follow requirements as directed by the CEM or PM; - Report any potential environmental issues to the CEM, including but not limited to diesel or oil spillages, uncontrolled waste storage, excessive dust generation, sewage holding tank overflow; - Carry out work in accordance with the requirements of this EMP; - Exercise due care, skill and judgment when carrying out tasks; and - Implement corrective actions which have been approved by the appointed site supervisor. - Attempt the Environmental Training |
| Contractor Project Manager | - Ensure resources are provided to prepare and implement the EMP; - Approve the EMP and any amendments to the EMP; and - Approve monthly reports or other reports about environmental issues and non-conformances to the Client, in the regular reporting structure and when an issue arises. - Apply for obtaining the permits and licenses concerning the Site activities, the Quarries, pit Borrow ecc |
| Construction Manager | - Control and monitor actions required by the EMP; - Monitor subcontractor performance and commitment; - Report all environmental issues to the Contractor’s Environmental Manager and Project Manager; - Maintain systems and records for waste management, sewage disposal, fuels and chemicals, and training; - Ensure documented procedures are followed, and records kept on site; - Ensure any complaints are passed onto the PM within 24 hours of receiving the complaint; and - The Site Engineer and/or the Contractor’s Environmental Manager will confirm the information on site and then report the matter to the Project Manager. |
| Contractor’s Environmental Manager | - Prepare, review and update this EMP; - Prepare monthly environmental reports to PM; - Ensure any changes to the EMP are made known to all staff and subcontractors; - Liaise with the Project Manager’s Environmental Representative to ensure variations to the scope or timing of the work that may impact on the environment are discussed, and be point of contact for all environmental issues and reporting; - Prepare information for environmental site training (toolbox talks); - Deliver or prepare information for environmental inductions; - Ensure materials being used on the site are environmentally friendly and safe; - Close-out all environmental non-conformances; - Conduct audits and inspections as required by the EMP at work sites; - Undertake confirmation of staff observations regarding non conformances; - Maintain the environmental training register; - Maintain the incidents and complaints register; - Be the one point of contact for discussing any changes to the scope of works, project boundary limits, reporting of incidents and non-conformances and other environmental issues. |
| Site Environmental Inspector | - Ensure the environmental controls in this EMP are being implemented across the contract site; - Ensuring changes to the EMP are communicated to staff; - Inform staff of environmental requirements; - Report all environmental issues to the Site Engineer and Contractor’s Environmental Representative; - Monitor subcontractor performance on work sites; - Communicate instructions or information (from the EMP or the Contractor’s Environmental Representative) to staff on site; - Conduct tool box talks containing environmental information; and - Report information to the Site Engineer and the Contractor’s Environmental Representative. |

Where a control measure used to limit environmental harm is found to be inadequate, or a potential incident is likely, the PM may request remedial works to be carried out.

Engineer and Project Manager have the authority to stop work immediately if any unacceptable impact on the environment is occurring or likely to occur.  The Client shall be immediately informed of the instruction to cease the  work.

The contact of Todini Personnel involved in the Environmental  Management is listened below:

| Company | Position | Name | Telephone | E mail |
| --- | --- | --- | --- | --- |
| TODINI | Project Manager | Smirnoff Eugeniy | 8 701 941 31 81 | carrel@ucr.net |
| TODINI | Safety Engineer | Boranbay Mananhaev | 8 702 693 30 42 |  |
| TODINI | Contractor’s Environmental Manager | Mundo Giancarlo | 8 702 820 54 74 | gmundo@todini.it |
| TODINI | Site Manager | Di Giammartino Cluadio | 8 701 537 00 56 |  |

  - Site Environmental Induction

The Contractor’s Environmental Representative will present the site environmental induction to new staff within seven (7) days \ their first day of work.  All site managers and relevant field staff.

The induction shall include the following minimum content:

- definition of “environment” that includes air, land, stormwater, groundwater, waste management, recycling, noise, plants and animals, and heritage items;

- Duty of care (responsibilities) – a statement that every worker has a responsibility to carry out their duties in an environmentally responsible way, and to report environmental harm or incidents, and to carry out their work in accordance with the responsibilities outlined in Section 2 of this EMP;

- EMP – purpose of the EMP and any specific EMP measures.  This may include some of the topics listed above;

- Environmental incident response and reporting – requirement to report incidents, incident response procedure;

- Waste management practices – minimise waste generated; sort and store recyclable waste separately; location of waste recycling areas and skip bins; duty to report overflowing waste bins;

- Fuel and chemical usage and storage practices – including location of spill clean-up kits, and general instructions on how to use;

- Specific environmental impacts that may arise from activities at the construction site, and the appropriate controls – for example:

  - Separating recycling waste from general waste;

  - Managing potential disturbance to nearby residential areas;

  - Covering truck loads of sand to limit dust;

  - Using drip trays under portable diesel generators;

  - Protection of green zones;

  - Draining oil filters prior to disposal; and

  - Disciplinary action, outcomes and penalties for failing to comply with environmental requirements.

  - Toolbox Talks

Tool box talks for all labourer and supervisory staff will be held every month, and will contain environmental updates or awareness.  Topics that will be covered in toolbox talks are:

- Specific issues / sections in EMP;

- Location of fuel spill clean-up materials, and procedures;

- Environmental incidents or recurring non-conformances; and

- Waste recycling.

Records will be kept of the environmental informations presented at toolbox talks, the date of training and the staff who received the training.

  - Training Records

A record of training attendance for the subcontractors will be kept for each training presentation, using the form attached in Annex 5.

All information recorded on the training attendance form will be transferred into a training register, which records all information from the training attendance form, and date for refresher training, and results of any assessment.  The training register template has been included in Annex 2. It is the responsibility of the Contractor’s Environmental Manager  to maintain the training register.  The Contractor’s Project Manager must receive updated copies of the training register.

  - EMERGENCIES AND COMPLAIN

Incidents

All environmental incidents will be responded to in a timely manner to stop or limit environmental harm or nuisance, and will be reported, investigated, remedied and documented.

Examples of environmental incidents include:

- Incorrect disposal of hazardous waste (such as chemical residues or waste oil);

- Covering contaminated soil with clean soil;

- Diesel spills (greater than 20 Litres) onto unsealed ground, during routine fuelling of trucks and vehicles;

- Discovery of heritage artefacts during earthworks.

  - General Incident Response Procedure

The general response procedure (refer Steps 1 – 5 below) will be used for all environmental incident:

Step 1	Safety.  Assess the safety hazards to the incident observer, other staff, other contractors and the public.  Do not carry out any unsafe actions when responding to environmental incidents – safety considerations must take precedence.

Safety considerations should include:

- Identify specific safety hazards such as dangerous chemicals, fire, toxic smoke from chemical fires, or injured wildlife;

- Close off areas to prevent traffic or public access;

- Locate and use personal protective equipment (PPE) such as gloves, glasses, helmet, safety footwear, or a lifting device;

- Take other actions as necessary to create safe conditions (for example, stop all smoking and remove ignition sources); and

- Consult the health and safety representative for further advice.

Step 2	Stop the source.  Where simple, ongoing incidents are continuing to cause environmental impacts, identify the cause, and if it is safe to do so, stop or limit the incident from occurring (for larger, dangerous or complex incidents go straight to Step 3), for example:

- Turn off pumps or isolate lines if pipes are leaking;

- Arrange for sewage holding tank pump-out if a sewage overflow has occurred;

- Stop earthworks where injury or death to animals is occurring; and

- Stop the use of excessively noisy equipment or noisy practices, particularly during the night time, if a noise complaint has been received.

Step 3	Report the incident.  Notify the relevant parties of the incident.

It is mandatory to notify the following parties immediately (after or as part of making the incident site safe):

- Notification to site management, including as per any EMP requirements.  If the EMP is not in place, notify the supervisor on duty and the contractor’s environmental representative (if this person did not observe or identify the incident); and

Step 4	Control the impacts.  Take immediate action (determine personnel, materials and equipment necessary) to prevent or limit environmental harm that has occurred or that may occur as a result of the incident.  For example:

- Contain free liquids, fuels or chemicals by draining into containers or pumping into mobile tanks or other containment vessels.  Do not pump any waste water or chemicals into stormwater connections unless directed by civil services;

- Deploy fuel spill clean-up materials (from spill kits), such as booms, absorbent particulate matter or absorbent mats; and

- Get extra waste skips delivered, for temporary storage of contaminated soil or other waste associated with the clean up.

If required seek help to manage the incident’s impacts and to implement control measures.  For example:

- For specialist environmental advice contact the contractor’s environmental representative or Nakheel Environment Department; and

- Fuel and chemical spills on public roads can present traffic hazards, call Civil Defence (Fire) on 997 for assistance in clean-up of spills on roads.

Step 5	Follow-up management.  When the incident is under control, and initial clean-up or prevention measures have been implemented, the contractor must complete the following:

- Monitor any controls that have been put in place for the incident, for example, absorbent materials used in fuel spill clean-up will require changing when fully saturated;

- Carry out an incident investigation and document it.  The basic cause of the incident must be identified, and the corresponding preventative actions determined;

- Monitor or re-inspect the area for any impacts.  Sometimes environmental impacts from incidents may become more apparent over time (for example, vegetation dying in an area over time due to changes in groundwater height or quality);

- Re-stock any materials used in incident response, such as personal protective equipment (PPE) or spill clean-up materials;

- Update the Environmental Incident Register with details of the incident, investigations and follow-up actions.  The following information must be included in the Environmental Incident Register:

- Date and time of incident or when incident was first brought to the attention of the contractor;

- Incident location;

- Incident details – what happened;

- Incident reported by and incident reported to;

- Immediate actions taken;

- Environmental Incident Report submitted to Engineer

- Corrective and preventative actions and who is responsible for these;

- Due date for corrective/ preventative actions; and

- Incident close-out date and sign-off.

Incidents will be reported using the form provided in Annex 3.

Information from incident reports and any updates will be entered into the Environmental Incident Register.  The template for the Incident Register is provided in Annex 3.

  - Incident and Emergency Preparedness

The following will be done to ensure that the staff are prepared for incidents and emergencies:

- Copies of incident response procedures will be displayed/ placed in higher-risk areas.  For example, the Fuel and Liquid Chemical Spill Response procedure will be placed in/ near spill response kits or bulk fuel storage facilities;

- Emergency response equipment will be kept in stock at the location where higher risk activities are being carried out.  The emergency response equipment must be labelled, and have simple directions for usage;

- Personal protective (safety) equipment (PPE) required for responding to emergencies, as well as other emergency response equipment, will be available at appropriate locations;

- Materials Safety Data Sheets (MSDS) will be kept for each chemical or fuel that is used, in an accessible location near the storage and usage areas;

- Emergency response training and awareness will be carried out for staff involved in activities that have higher environmental risk, including the use of emergency response equipment.  Training details must be recorded in the Training Register (refer Section 4); and

- Emergency response drills will be carried out as part of training.

A copy of the Material Safety Data Sheets (MSDS) for all the chemicals used on the project site will also be kept at each site office.

  - Complaints Handling

All complaints from the public or neighbors relating to environmental matters will be managed as follows:

- Details of the complaint will be recorded in a complaints register, and updated as necessary;

- The complaint will be investigated.  The investigation will identify appropriate corrective actions, the responsible parties/person to complete the corrective actions, and due date for completion;

- The complainant will be updated regularly, until the time follow-up action on a complaint is deemed complete; and

- The complaint will be signed-off as complete by the contractor’s project manager, once the matter has been satisfactorily addressed.

The complaints register will record the following details:

- Date and time of complaint;

- Name of person/ company making the complaint;

- Contact telephone number for the complainant;

- Details of the complaint;

- Details of action taken; and

- Time/date that the complainant is contacted and given follow-up information/complaint closed-out.

The complaints register template is included in Annex 3

# Annexes

  - List of Annex

Annex 1		Site Key Plan and Pit Borrow

Annex 2		Environmental Induction Forms

Annex 3		Emergency and Compliance Forms

Annex 4		License of the Environmental Laboratory